# EENG2131 - Lab 6 – Digital Clock

Antonio Veguilla Hernandez

Student ID: P000166790

For this lab, we’ll be creating a Digital Clock using the Seven-Segment displays on the Basys3 FPGA dev board. We’ll have the left two digits represent the Hours and the right two digits represent the Minutes. We’ll use the right-most decimal point to flash every second.

| # Part 1 –Clock Divider Bring back your Clock Divider module that produced 1 Hz output. Simplify the module to have just one output but change it to be a “once every minute” signal, or 1/60 Hz. Connect it to one of the LEDs on the FPGA to prove that it’s working. My Verilog module module Clock1min(clk, clk_1min, clk_1sec); input clk; // Standard 100MHz clock output reg clk_1min = 0; //60Hz clock output = 1 minute output reg clk_1sec = 0; // 1Hz clock output = 1 second reg [39:0] count10 = 0; //counter with a magnitude of 2^40 reg [28:0] count11 = 0; //counter with magnitude of 2^29. always @(posedge clk) begin if(count11 == 50000000) //1second// begin clk_1sec = ~clk_1sec; count11 <=0; end else count11 <= count11 + 1; if(count10 == 3000000000) //1minute// begin clk_1min = ~clk_1min; count10 <= 0; end else begin count10 <= count10 + 1; end en endmodule # Part 2 – Enhance SevenSegmentFourDigits module to support DP Since we ignored the decimal point (DP) last time, we need to enhance our SevenSegmentFourDigits module to support the decimal points: - Expand the segN signal to be 8-bit (that is, [7:0]) with the upper bit controlling the decimal point). | - Add a new 4-bit input called DPs that will indicate which decimal points should be lit. This should be an active-high signal, so you may need to invert the values before driving the segN signal. - Add the necessary logic to select the correct bit of DPs to be output as part of segN. My Verilog Module module SevenSegmentFourDigits(clk, bcd0, bcd1, bcd2, bcd3, AN, segN, DPs); input clk; //Standard 100MHz clock input [3:0]bcd0; //right most digit input [3:0]bcd1; input [3:0]bcd2; input [3:0]bcd3; //left-most digit input [3:0]DPs; //decimal point control, active high output [3:0]AN; //digit enable active low output [7:0]segN; //segment enable, active low BCDtoSevenSegN dut(trueBCD, segN[6:0]); DigitController dut1(clk, AN); reg [3:0]trueBCD = 0; reg segN7; assign segN[7] = segN7; always@(AN) begin if(AN[0] == 0) begin trueBCD = bcd0; segN7 = ~DPs[0]; end else if(AN[1] == 0) begin trueBCD = bcd1; segN7 = ~DPs[1]; end else if(AN[2] == 0) begin trueBCD = bcd2; segN7 = ~DPs[2]; end else if(AN[3] == 0) begin trueBCD = bcd3; segN7 = ~DPs[3]; end end endmodule |
| --- | --- |

| # Part 3 – BCD Counter BCD values typically range from 0-9, but with a clock, some of the digits need to “roll over” at 6! Let’s design a 4-bit BCD counter that has a parameter for the roll-over number. My Verilog Module module BCD_Counter(clk, resetN, value, rollover); parameter maxValue = 9; parameter resetVal = 0; input clk; //counter should increment at each clk posedge input resetN; //reset, active low, synchronous to clk output reg [3:0]value = 0; //counter value output reg rollover = 0; //binary output to indicate roll-over always @(posedge clk, negedge resetN) begin if(!resetN) begin value <= resetVal; rollover <= 0; end else if(value == maxValue) begin rollover <= 1; value <= 0; end else begin value <= value +1; rollover <= 0; end end endmodule My Verilog Tesbench Module module testbench_BCD_Counter; reg clk, resetN; wire [3:0]value; wire rollover; BCD_Counter dut1_1(clk, resetN, value, rollover); initial begin clk = 0; resetN = 1; #10 clk = 1; //1st posedge #10 clk = 0; #10 clk = 1; //2nd posedge #10 clk = 0; #10 clk = 1; //3rd posedge #10 clk = 0; #10 clk = 1; //4th posedge | #10 clk = 0; #10 clk = 1; //5th posedge #10 clk = 0; #10 clk = 1; //6th posedge #10 clk = 0; #10 clk = 1; //7th posedge #10 clk = 0; #10 clk = 1; //8th posedge #10 clk = 0; #10 clk = 1; //9th posedge #10 clk = 0; #10 clk = 1; //10th posedge #10 clk = 0; #10 clk = 1; //11th posedge #10 clk = 0; #10 clk = 1; //12th posedge #10 clk = 0; #10 clk = 1; //13th posedge #10 clk = 0; #10 clk = 1; //14th posedge #10 clk = 0; #10 clk = 1; //15th posedge #10 clk = 0; #10 clk = 1; //16th posedge #10 clk = 0; #10 clk = 1; //17th posedge #10 clk = 0; #10 clk = 1; //18th posedge |
| --- | --- |

| #10 clk = 0; #10 clk = 1; //19th posedge #10 clk = 0; #10 clk = 1; //20th posedge #10 //////Sequence to switch resetN///////////////// clk = 0; #10 clk = 1; //1st posedge resetN = 0; #10 clk = 0; #10 clk = 1; //2nd posedge #10 clk = 0; #10 clk = 1; //3rd posedge #10 clk = 0; #10 clk = 1; //4th posedge #10 clk = 0; #10 clk = 1; //5th posedge #10 clk = 0; #10 clk = 1; //6th posedge resetN = 1; #10 clk = 0; #10 clk = 1; //7th posedge #10 clk = 0; #10 clk = 1; //8th posedge #10 clk = 0; #10 clk = 1; //9th posedge #10 clk = 0; #10 clk = 1; //10th posedge #10 clk = 0; #10 clk = 1; //11th posedge resetN = 0; #10 clk = 0; | #10 clk = 0; #10 clk = 1; //12th posedge #10 clk = 0; #10 clk = 1; //13th posedge #10 clk = 0; #10 clk = 1; //14th posedge #10 clk = 0; #10 clk = 1; //15th posedge #10 clk = 0; #10 clk = 1; //16th posedge resetN =1; #10 clk = 0; #10 clk = 1; //17th posedge #10 clk = 0; #10 clk = 1; //18th posedge #10 clk = 0; #10 clk = 1; //19th posedge #10 clk = 0; #10 clk = 1; //20th posedge #10 clk = 0; #10 clk = 1; //21st posedge resetN =1; #10; end endmodule |
| --- | --- |

| # Part 4 – Hours:Minutes Clock Create a module to act as an Hours and Minutes clock display, with the following interface: My Verilog Module module ClockHoursMinutes(clk, reset, AN, segN); input clk; input reset; //Resets Active High output [3:0]AN; //digit enable, active low output [7:0] segN; //segment enable, active low reg resetN; wire [3:0]bcd0; wire [3:0]bcd1; wire [3:0]bcd2; wire [3:0]bcd3; /////////////////Instatiate//////////////////// SevenSegmentFourDigits(clk, bcd0, bcd1, bcd2, bcd3, AN, segN, DPs); Clock1min dut_clk(clk, clk_1min, clk_1sec); ////////////////////////////////////////////// wire clk_min; wire clk_1sec; wire [3:0]DPs; assign DPs[2] = clk_1sec; //This will make the DP between HRs & mins to blink every second BCD_Counter #(.resetVal(0), .maxValue(9))cnt1(clk_1min, resetN, bcd0, rollover0); BCD_Counter #(.resetVal(0), .maxValue(5))cnt2(rollover0, resetN, bcd1, rollover1); BCD_Counter #(.resetVal(1), .maxValue(9))cnt3(rollover1, resetN, bcd2, rollover3); BCD_Counter #(.resetVal(0), .maxValue(1))cnt4(rollover3, resetN, bcd3, rollover4); /////////////////////////////////////////////// always begin if(bcd3 == 1 & bcd2 == 2 & bcd1 == 5 & bcd0 == 9) begin resetN = 0; end else if(reset) resetN = 0; else resetN = 1; end endmodule My Verilog FPGA Interface module module FPGA_Interface(sw,an,seg,clk,led,btnC); output [3:0]an; input clk; input [15:0]sw; input btnC; output [7:0]seg; output [15:0]led; ClockHoursMinutes(clk, btnC, an, seg); endmodule | Constrain File ## This file is a general .xdc for the Basys3 rev B board ## To use it in a project: ## - uncomment the lines corresponding to used pins ## - rename the used ports (in each line, after get_ports) according to the top level signal names in the project ## Clock signal set_property PACKAGE_PIN W5 [get_ports clk] set_property IOSTANDARD LVCMOS33 [get_ports clk] create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk] ## Switches set_property PACKAGE_PIN V17 [get_ports {sw[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[0]}] set_property PACKAGE_PIN V16 [get_ports {sw[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[1]}] set_property PACKAGE_PIN W16 [get_ports {sw[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[2]}] set_property PACKAGE_PIN W17 [get_ports {sw[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[3]}] set_property PACKAGE_PIN W15 [get_ports {sw[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[4]}] set_property PACKAGE_PIN V15 [get_ports {sw[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[5]}] set_property PACKAGE_PIN W14 [get_ports {sw[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[6]}] set_property PACKAGE_PIN W13 [get_ports {sw[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[7]}] set_property PACKAGE_PIN V2 [get_ports {sw[8]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[8]}] set_property PACKAGE_PIN T3 [get_ports {sw[9]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[9]}] set_property PACKAGE_PIN T2 [get_ports {sw[10]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[10]}] set_property PACKAGE_PIN R3 [get_ports {sw[11]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[11]}] set_property PACKAGE_PIN W2 [get_ports {sw[12]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[12]}] set_property PACKAGE_PIN U1 [get_ports {sw[13]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[13]}] set_property PACKAGE_PIN T1 [get_ports {sw[14]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[14]}] set_property PACKAGE_PIN R2 [get_ports {sw[15]}] set_property IOSTANDARD LVCMOS33 [get_ports {sw[15]}] ## LEDs set_property PACKAGE_PIN U16 [get_ports {led[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {led[0]}] set_property PACKAGE_PIN E19 [get_ports {led[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {led[1]}] set_property PACKAGE_PIN U19 [get_ports {led[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {led[2]}] set_property PACKAGE_PIN V19 [get_ports {led[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {led[3]}] |
| --- | --- |

| ### set_property PACKAGE_PIN W18 [get_ports {led[4]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[4]}] ### set_property PACKAGE_PIN U15 [get_ports {led[5]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[5]}] ### set_property PACKAGE_PIN U14 [get_ports {led[6]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[6]}] ### set_property PACKAGE_PIN V14 [get_ports {led[7]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[7]}] ### set_property PACKAGE_PIN V13 [get_ports {led[8]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[8]}] ### set_property PACKAGE_PIN V3 [get_ports {led[9]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[9]}] ### set_property PACKAGE_PIN W3 [get_ports {led[10]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[10]}] ### set_property PACKAGE_PIN U3 [get_ports {led[11]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[11]}] ### set_property PACKAGE_PIN P3 [get_ports {led[12]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[12]}] ### set_property PACKAGE_PIN N3 [get_ports {led[13]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[13]}] ### set_property PACKAGE_PIN P1 [get_ports {led[14]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[14]}] ### set_property PACKAGE_PIN L1 [get_ports {led[15]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {led[15]}] ### ##7 segment display ### set_property PACKAGE_PIN W7 [get_ports {seg[0]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {seg[0]}] ### set_property PACKAGE_PIN W6 [get_ports {seg[1]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {seg[1]}] ### set_property PACKAGE_PIN U8 [get_ports {seg[2]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {seg[2]}] ### set_property PACKAGE_PIN V8 [get_ports {seg[3]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {seg[3]}] ### set_property PACKAGE_PIN U5 [get_ports {seg[4]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {seg[4]}] | ### set_property PACKAGE_PIN V5 [get_ports {seg[5]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {seg[5]}] ### set_property PACKAGE_PIN U7 [get_ports {seg[6]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {seg[6]}] ### set_property PACKAGE_PIN V7 [get_ports seg[7]] ### set_property IOSTANDARD LVCMOS33 [get_ports seg[7]] ### set_property PACKAGE_PIN U2 [get_ports {an[0]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}] ### set_property PACKAGE_PIN U4 [get_ports {an[1]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}] ### set_property PACKAGE_PIN V4 [get_ports {an[2]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}] ### set_property PACKAGE_PIN W4 [get_ports {an[3]}] ### set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}] ### ##Buttons ### set_property PACKAGE_PIN U18 [get_ports btnC] ### set_property IOSTANDARD LVCMOS33 [get_ports btnC] ### #set_property PACKAGE_PIN T18 [get_ports btnU] ### #set_property IOSTANDARD LVCMOS33 [get_ports btnU] ### #set_property PACKAGE_PIN W19 [get_ports btnL] ### #set_property IOSTANDARD LVCMOS33 [get_ports btnL] ### #set_property PACKAGE_PIN T17 [get_ports btnR] ### #set_property IOSTANDARD LVCMOS33 [get_ports btnR] ### #set_property PACKAGE_PIN U17 [get_ports btnD] ### #set_property IOSTANDARD LVCMOS33 [get_ports btnD] ### ##Pmod Header JA ### ##Sch name = JA1 ### #set_property PACKAGE_PIN J1 [get_ports {JA[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[0]}] |
| --- | --- |

| ### ##Sch name = JA2 ### #set_property PACKAGE_PIN L2 [get_ports {JA[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[1]}] ### ##Sch name = JA3 ### #set_property PACKAGE_PIN J2 [get_ports {JA[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[2]}] ### ##Sch name = JA4 ### #set_property PACKAGE_PIN G2 [get_ports {JA[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[3]}] ### ##Sch name = JA7 ### #set_property PACKAGE_PIN H1 [get_ports {JA[4]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[4]}] ### ##Sch name = JA8 ### #set_property PACKAGE_PIN K2 [get_ports {JA[5]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[5]}] ### ##Sch name = JA9 ### #set_property PACKAGE_PIN H2 [get_ports {JA[6]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[6]}] ### ##Sch name = JA10 ### #set_property PACKAGE_PIN G3 [get_ports {JA[7]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JA[7]}] ### ##Pmod Header JB ### ##Sch name = JB1 ### #set_property PACKAGE_PIN A14 [get_ports {JB[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[0]}] ### ##Sch name = JB2 ### #set_property PACKAGE_PIN A16 [get_ports {JB[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[1]}] ### ##Sch name = JB3 ### #set_property PACKAGE_PIN B15 [get_ports {JB[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[2]}] ### ##Sch name = JB4 | ### #set_property PACKAGE_PIN B16 [get_ports {JB[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[3]}] ### ##Sch name = JB7 ### #set_property PACKAGE_PIN A15 [get_ports {JB[4]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[4]}] ### ##Sch name = JB8 ### #set_property PACKAGE_PIN A17 [get_ports {JB[5]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[5]}] ### ##Sch name = JB9 ### #set_property PACKAGE_PIN C15 [get_ports {JB[6]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[6]}] ### ##Sch name = JB10 ### #set_property PACKAGE_PIN C16 [get_ports {JB[7]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JB[7]}] ### ##Pmod Header JC ### ##Sch name = JC1 ### #set_property PACKAGE_PIN K17 [get_ports {JC[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[0]}] ### ##Sch name = JC2 ### #set_property PACKAGE_PIN M18 [get_ports {JC[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[1]}] ### ##Sch name = JC3 ### #set_property PACKAGE_PIN N17 [get_ports {JC[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[2]}] ### ##Sch name = JC4 ### #set_property PACKAGE_PIN P18 [get_ports {JC[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[3]}] ### ##Sch name = JC7 ### #set_property PACKAGE_PIN L17 [get_ports {JC[4]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[4]}] ### ##Sch name = JC8 ### #set_property PACKAGE_PIN M19 [get_ports {JC[5]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[5]}] |
| --- | --- |

| ### ##Sch name = JC9 ### #set_property PACKAGE_PIN P17 [get_ports {JC[6]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[6]}] ### ##Sch name = JC10 ### #set_property PACKAGE_PIN R18 [get_ports {JC[7]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JC[7]}] ### ##Pmod Header JXADC ### ##Sch name = XA1_P ### #set_property PACKAGE_PIN J3 [get_ports {JXADC[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[0]}] ### ##Sch name = XA2_P ### #set_property PACKAGE_PIN L3 [get_ports {JXADC[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[1]}] ### ##Sch name = XA3_P ### #set_property PACKAGE_PIN M2 [get_ports {JXADC[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[2]}] ### ##Sch name = XA4_P ### #set_property PACKAGE_PIN N2 [get_ports {JXADC[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[3]}] ### ##Sch name = XA1_N ### #set_property PACKAGE_PIN K3 [get_ports {JXADC[4]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[4]}] ### ##Sch name = XA2_N ### #set_property PACKAGE_PIN M3 [get_ports {JXADC[5]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[5]}] ### ##Sch name = XA3_N ### #set_property PACKAGE_PIN M1 [get_ports {JXADC[6]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[6]}] | ### ##Sch name = XA4_N ### #set_property PACKAGE_PIN N1 [get_ports {JXADC[7]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {JXADC[7]}] ### ##VGA Connector ### #set_property PACKAGE_PIN G19 [get_ports {vgaRed[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[0]}] ### #set_property PACKAGE_PIN H19 [get_ports {vgaRed[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[1]}] ### #set_property PACKAGE_PIN J19 [get_ports {vgaRed[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[2]}] ### #set_property PACKAGE_PIN N19 [get_ports {vgaRed[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaRed[3]}] ### #set_property PACKAGE_PIN N18 [get_ports {vgaBlue[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[0]}] ### #set_property PACKAGE_PIN L18 [get_ports {vgaBlue[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[1]}] ### #set_property PACKAGE_PIN K18 [get_ports {vgaBlue[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[2]}] ### #set_property PACKAGE_PIN J18 [get_ports {vgaBlue[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaBlue[3]}] ### #set_property PACKAGE_PIN J17 [get_ports {vgaGreen[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[0]}] ### #set_property PACKAGE_PIN H17 [get_ports {vgaGreen[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[1]}] ### #set_property PACKAGE_PIN G17 [get_ports {vgaGreen[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[2]}] |
| --- | --- |

| ### #set_property PACKAGE_PIN D17 [get_ports {vgaGreen[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {vgaGreen[3]}] ### #set_property PACKAGE_PIN P19 [get_ports Hsync] ### #set_property IOSTANDARD LVCMOS33 [get_ports Hsync] ### #set_property PACKAGE_PIN R19 [get_ports Vsync] ### #set_property IOSTANDARD LVCMOS33 [get_ports Vsync] ### ##USB-RS232 Interface ### #set_property PACKAGE_PIN B18 [get_ports RsRx] ### #set_property IOSTANDARD LVCMOS33 [get_ports RsRx] ### #set_property PACKAGE_PIN A18 [get_ports RsTx] ### #set_property IOSTANDARD LVCMOS33 [get_ports RsTx] ### ##USB HID (PS/2) ### #set_property PACKAGE_PIN C17 [get_ports PS2Clk] ### #set_property IOSTANDARD LVCMOS33 [get_ports PS2Clk] ### #set_property PULLUP true [get_ports PS2Clk] ### #set_property PACKAGE_PIN B17 [get_ports PS2Data] ### #set_property IOSTANDARD LVCMOS33 [get_ports PS2Data] ### #set_property PULLUP true [get_ports PS2Data] ### ##Quad SPI Flash ### ##Note that CCLK_0 cannot be placed in 7 series devices. You can access it using the ### ##STARTUPE2 primitive. ### #set_property PACKAGE_PIN D18 [get_ports {QspiDB[0]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[0]}] ### #set_property PACKAGE_PIN D19 [get_ports {QspiDB[1]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[1]}] ### #set_property PACKAGE_PIN G18 [get_ports {QspiDB[2]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[2]}] | ### #set_property PACKAGE_PIN F18 [get_ports {QspiDB[3]}] ### #set_property IOSTANDARD LVCMOS33 [get_ports {QspiDB[3]}] ### #set_property PACKAGE_PIN K19 [get_ports QspiCSn] ### #set_property IOSTANDARD LVCMOS33 [get_ports QspiCSn] ### ## Configuration options, can be used for all designs ### set_property CONFIG_VOLTAGE 3.3 [current_design] ### set_property CFGBVS VCCO [current_design] ### Conclusion: ### The laboratory was very challenging, but at the same time interesting and full of surprises. I did really enjoy seeing the 7-segment digit display transformed into a clock. It took a lot of effort to make it work, but at the end it was worth it. ### An appendix section was included to put the screenshots of the testbench simulations. |
| --- | --- |

### BCD to Seven Segment Testbench Simulation

### Digit Controller Testbench Simulation

### BCD Counter Testbench Simulation